[MOPSRV.DOC]                                                   January 1991

RSTS/E Version V10.0-L                           MOP Server Version V1.1-12

RSTS/E MOP Server

Written by

Terry Kennedy

Operations Manager, Academic Computing

Saint Peter's College

2641 Kennedy Blvd

Jersey City, NJ 07306

(201) 435-0252

terry@spcvxa.spc.edu

terry@spcvxa.bitnet

OVERVIEW:

RSTS/E MOP Server performs the following functions:

o    Provides  the  operating software for DECservers upon request  by  the

server.

o    Receives crash dumps from DECservers upon request by the server.  Such

crash  dumps  are in the proper format for analysis  by  DEC  software

support.

o    Logs  all  requested and successful load/dump requests to  the  system

console (in DECnet/E event format) and to a log file.

RSTS/E  MOP  Server  can be configured to accept or  reject  requests  from

servers not in the configuration file. This can be used to restrict loading

to  a  subset of the servers configured on the  Ethernet.  Additionally,  a

user-configurable  delay is available so that a supported load host  has  a

chance  to load the server first, with RSTS/E MOP Server assuming a  backup

mode of operation. The default load image name for the server is  supported

via a mapping table which maps the image name requested by the server  into

a valid RSTS/E file specification.

INSTALLATION INSTRUCTIONS:

Network   distribution   is via mailing of the sources  and  documentation.

Thus,  network distribution does not contain object or executables. If  you

receive  a  tape, there will be a single directory containing all  the  re-

quired files.

Steps necessary to install mopsrv:

o    Copy the entire distribution into a disk directory. This can be a /NO-

USER  account.  I suggest one of  the reserved-for-customer-use  [0,*]

directories.

o    Define the MOP$ logical:

$ ASSIGN/SYSTEM _SY:[somewhere] MOP$:

o    If you received the distribution as ASCII text (no binaries included),

you will need to create the MOPSRV.TSK programs. To do this, you  will

need the Basic-Plus-2 Version 2 compiler. This is done by invoking the

MOPSRV.COM procedure:

$ @MOP$:MOPSRV

You NEED NOT do this step if you received your distribution on tape or

via DECnet.

o    Edit  the file MOP$:MOPSRV.CNF to  tailor the configuration  for  your

site.  This is where you select the supported servers, etc. A configu-

ration  entry consists of a keyword letter, followed by a space,  fol-

lowed  by  a  value for the keyword. A sample  configuration  file  is

supplied  on  the distribution kit. A brief discussion  of  each  item

follows:

H - Host system DECnet address and name. This information is passed to

the  server when loading is complete. The host name is displayable  by

issuing the server command "SHO SERVER STATUS". If this host is not on

a DECnet, use 0.0 as the host address and some convenient  identifica-

tion for the host name.

Example: H 3.4 SPC11D

S - Server identification record. The first field is the server Ether-

net  address  in octet format. This information appears on  a  barcode

label  near the Ethernet connector on the server. The second field  is

the  DECnet node number of the server. The third field is  the  DECnet

node name of the server. The node number field is for logging purposes

only.  The node name field is used for logging, as well as for  naming

crash dump files received from the server.

Example: S 08-00-2B-05-E5-C4 2.1 LT2001

W - Wildcard load flag.  Can be either Y or N. If Y, all servers  will

receive  load/dump  support regardless of whether they appear in  a  S

configuration record or not. If N, servers not listed in an S configu-

ration  record will not receive load/dump support from this  host.  If

set  to Y and the server is not listed in an S  configuration  record,

crash  dumps will be stored using the last three octets of the  server

Ethernet address as the filename.

Example: W Y

M - Load image mapping. The first field is the name of the load  image

as requested by the server. The second field is the RSTS/E file speci-

fication for the load image. This mapping is needed since server  load

images are typically 9 character names which may not be unique in  the

first 6 positions. The RSTS/E file specification may contain a  device

and/or PPN specification, or may contain a logical name. It must be  a

file  which exists on the local node. Typically, the load  images  are

stored in the MOP$: directory as well.

Note  that  servers such as the DECserver 550 do not  specify  a  load

image name when requesting a load. In such a case, MOPSRV will  fabri-

cate a load image name consisting of the server Ethernet address.  For

example, a server with Ethernet address 08-00-2B-11-22-33 would appear

to be requesting image 08002B112233.SYS, which would then be translat-

ed  through  an M record to a RSTS/E file  specification.  Thus,  each

server can have a unique load image, or they can be mapped to a single

load image.

Example: M PR0801ENG.SYS MOP$:DS200.SYS

D  - Delay before responding.  Delay the specified number  of  seconds

before  responding  to a load/dump solicit message. This gives  a  VAX

system  time  to respond. Useful if you wish to normally load  from  a

supported host, but want to use the RSTS/E system as a backup when the

VAX  is  down. A value of at least 15 is needed to allow  the  VAX  to

reliably respond. Values greater than 30 are not generally useful,  as

the server will time out the request after 30 seconds. Note that  this

delay  is only made when a load/dump solicit message is received -  if

the request was made explicitly to this host, no delay will be used.

Example: D 0

E  -  Specify an alternate Ethernet device, protocol, or  block  size.

This parameter allows the user to override the default Ethernet param-

eters.  This  function  should never be needed, but  is  provided  for

debugging purposes. For more information, consult the program source.

L - Defines the logging level desired. Level 0 logs all events, and is

suggested  for  your  initial installation. Level 1  logs  all  events

except  load/dump solicit messages. Any load/dump processing  by  this

system will still be logged. This is a good value to use if you have a

busy  network  with lots of servers loading (and  dumping?)  that  you

don't want to know about. If you select this level, you whould  proba-

bly  turn off wildcard load/dump support (with "W N") or  this  server

will  try to load/dump anyway. Level 2 supresses all logging  informa-

tion.

Example: L 0

Q - Terminates the scan of the configuration file.

Example: Q

o    Ensure that the protection code of MOPSRV.TSK is <232>, as follows:

$ PIP MOP$:MOPSRV.TSK<232>/RE

o    Edit the system startup file [0,1]START.COM to assign the MOP$ logical

and start the MOPSRV job running:

ASSIGN/SYSTEM _SY:[somewhere] MOP$:

RUN MOP$:MOPSRV

o    Load  the  DECserver software appropriate to your server(s)  into  the

location and filename specified by the configuration file M record(s).

If  you are running RSTS/E Version 10.0 or higher, you can simply  use

BACKUP  to restore the load image from the VMS DECserver load kit  and

then  rename  it.  If you are running an earlier  version  of  RSTS/E,

BACKUP will not be able to directly read the VMS DECserver load kit as

the block size is too large. You will have to make other  arrangements

to obtain a RSTS/E copy of the load kit.

[NOTE: The license for the server software is  included

with  the  server  when you purchase  the  server  from

Digital.  No license is required to purchase  a  server

load kit. If you have any questions about the licensing

of  server software, please contact your local  Digital

office.]

MOPSRV EVENT LOGGING:

MOPSRV  records  load/dump messages to the system  console  (KB0:)  in

DECnet/E  message  format.  These messages are also stored  in  the  MOPSRV

logging file, MOP$:MOPSRV.LOG.

Solicit messages will always be logged, regardless of whether the MOP

Server is configured to load/dump the server making the request and regard-

less  of the presence/absence of the appropriate load file. This is  useful

for determining the server address or load image name for a server you want

to add load/dump support for. A sample Solicit message is shown below:

Event type 0.3, Automatic line service

Occurred 06-Jul-90 03:14 AM on node 3.4 (SPC11D)

Circuit UNA-0, Load, Solicited, Node = 2.1 (LT2001)

File = PR0801ENG.SYS, Operating system

Ethernet address = 08-00-2B-05-E5-C4

Request messages are logged when the DECserver selects this MOP Server

host  as  the host to perform the load/dump operation.  Load  requests  are

always preceded by a solicit; dump requests may not be.

Event type 0.3, Automatic line service

Occurred 06-Jul-90 02:48 AM on node 3.4 (SPC11D)

Circuit UNA-0, Load, Requested, Node = 2.6 (LT2006)

File = PR0801ENG.SYS, Operating system

Ethernet address = 08-00-2B-0A-FD-30

Success messages are logged when a load or dump operation has success-

fully  completed. As soon as the MOP Server has displayed the Success  mes-

sage it is free to process requests from another server.

Event type 0.3, Automatic line service

Occurred 06-Jul-90 02:48 AM on node 3.4 (SPC11D)

Circuit UNA-0, Load, Successful, Node = 2.6 (LT2006)

File = PR0801ENG.SYS, Operating system

Ethernet address = 08-00-2B-0A-FD-30

ERROR CONDITIONS:

The MOP Server will normally be idle in an XE or XH state. If you  see

it  in  an  HB state or receive DECnet event 5.15 (Receive  failed  -  user

buffer  unavailable) messages, the server has died for some reason. If  you

ATTACH to the MOPSRV job, you will receive an error message which should be

forwarded  to  me for analysis. During development  I  experienced  several

"Device hung or write locked" messages when trying to communicate with  the

Ethernet  portal.  Since  the documentation states that  this  indicates  a

system  hardware  or  software problem, and it was  repeatable  on  several

different  processors,  I suspect some problems with  the  RSTS/E  Ethernet

driver. I have not seen any of these problems since the code was completed,

however.  If  the server encounters an unexpected error, it  will  write  a

small  amount  of debugging information to the file  MOP$:MOPSRV.DMP.  When

reporting a problem, please include the information in that file. Note that

the MOPSRV dump file is entirely different from terminal server dump files,

which are also stored in the MOP$: directory.

RESTRICTIONS:

The  MOP Server is single-threaded, unlike the server on VAX/VMS  sys-

tems. Thus, if a number of servers simultaneously request service (such  as

after  a  power failure), some of them may give timeout messages.  The  MOP

server  will get around to loading them after it has finished it's  current

request.  With 12 servers (DS200/250 mix), all servers were  simultaneously

initialized.  Within  5 minutes, all were re-loaded. If you  have  multiple

RSTS  hosts running the MOP Server available, they will automatically  dis-

tribute the loading task equitably.

There is no support for configuring server load images which are  only

host-configurable, such as the DECserver 550 software. The load image  will

have  to  be  configured on an appropriate host and then  loaded  onto  the

RSTS/E  system(s) for downloading. Further, it is important to remember  to

update  all  hosts which load such servers, in order to have  a  consistent

configuration, whenever the load image is modified or updated.

KNOWN PROBLEMS:

o    This  is  Version 1.1. All problems reported against  V1.0  have  been

corrected.  Feel free to report new bugs or desired features to me for

future work.

OPTIONAL SOFTWARE:

An  additional  program named CNODE is also supplied on  the  kit.  It

provides  functionality similar to the VMS CONNECT NODE command. When  run,

it  prompts the user for the name of the node to connect to, which must  be

in  the  MOPSRV.CNF  configuration file. It then attempts  to  establish  a

remote  console session to the server's virtual management console. If  the

console  is in use by another user, an error message is  displayed.  Other-

wise, the user is connected to the console. Pressing Control-D will  termi-

nate your remote console connection.

CNODE  may be defined as a system command (CCL), in which case the  serv-

er's node name may be specified on the command line.

CNODE may be rebuilt by executing the command:

$ @MOP$:CNODE

Note  that CNODE (like MOPSRV) is single-threaded. However, while  MOPSRV

will  only  respond to external load/dump requests, you can  use  CNODE  to

attempt to connect to a server that doesn't exist. This will cause your job

to  hang. To recover, log into another port and use the REMOVE/JOB  command

to remove the hung CNODE job. The reason this occurs is because the  RSTS/E

Ethernet driver does not have a "read with timeout" function, and thus  the

job  stalls in an XE (or XH) state. It was felt that the occasional  incon-

venience outweighed the system impact of using no-wait Ethernet reads.